Types of number system

- Briefly explain the following representations: sign magnitude, twos complement, biased.

- Explain how to determine if a number is negative in the following representations: sign magnitude, twos complement, biased.

- What is the sign-extension rule for twos complement numbers?

- How can you form the negation of an integer in twos complement representation?

- In general terms, when does the twos complement operation on an n-bit integer produce the same integer?

- What is the difference between the twos complement representation of a number and the twos complement of a number?

- If we treat two twos complement numbers as unsigned integers for purposes of addi tion, the result is correct if interpreted as a twos complement number. This is not true for multiplication. Why?

- What are the four essential elements of a number in floating-point notation?

- What is the benefit of using biased representation for the exponent portion of a float ing-point number?

- What are the differences among positive overflow, exponent overflow, and significand overflow?

- What are the basic elements of floating-point addition and subtraction? 10.12 Give a reason for the use of guard bits.

- List four alternative methods of rounding the result of a floating-point operation.

- Represent the following decimal numbers in both binary sign/magnitude and twos complement using 16 bits: + 512; - 29.

- Represent the following twos complement values in decimal: 1101011; 0101101.